ROZITEK

RCA Line Feeding System

System Architecture & Technical Documentation

ESP32-S3 Beeper  •  RCA Middleware  •  PDA Kitting App

HIKROBOT RCS-2000 V3.3 Integration  •  SQLite Database

Version 3.8

March 2026

# Document Information

| Field | Value |
| --- | --- |
| Title | RCA Line Feeding System — Architecture & Technical Stack |
| Version | 3.8 |
| Author | Rozitek Solutions & Sales |
| Date | March 2026 |
| Classification | Internal / Customer |

# Table of Contents

# 1. Executive Summary

The RCA Line Feeding System automates material replenishment in factory environments using AMR robots. It manages 19 workstations across 3 kitting zones with two integrated workflows: physical button-call panels (ESP32-S3) and a mobile PDA kitting dispatch application (Hikvision DS-MDT302).

The system integrates with HIKROBOT RCS-2000 V3.3 via REST API, supporting two task types: MF01 (6-position full cycle triggered by button press) and MF02 (2-position kitting dispatch triggered by PDA scan). All dispatch history is persisted in a SQLite database for tracking, audit, and operational continuity.

## 1.1 Key Capabilities

- 19 stations with ESP32-S3 hardware panels: physical button, status LED, buzzer, AP mode recovery

- PDA web app with hardware QR scanner support, keypad navigation, and auto-advance workflow

- Two task types: MF01 (button-call, 6 positions) and MF02 (PDA dispatch, 2 positions)

- SQLite database: dispatch history, state sync with RCS, PDA cancel, position/station blocking

- Flow-based admin dashboard with rack usage time tracking and live station status

- Zero dependencies — Python 3.6+ stdlib only, deployable on air-gapped HikvisionOS servers

- Production-stable: ThreadedHTTPServer, watchdog restart, full exception handling, DB orphan sync

# 2. System Architecture

## 2.1 Component Overview

| Tier | Component | Responsibility |
| --- | --- | --- |
| Edge (19x) | ESP32-S3 Beeper | Button input, LED/buzzer feedback, WiFi STA+AP, FreeRTOS dual-core |
| Edge (Nx) | DS-MDT302 PDA | QR scan, kitting dispatch wizard, keypad nav, dispatch history+cancel |
| Server | RCA Middleware | State machine, SQLite DB, RCS API, dashboard, PDA server, anti-spam |
| Server | RCS-2000 V3.3 | AMR fleet scheduling, task execution, route planning, agvCallback |

## 2.2 Network Topology

| Component | Address |
| --- | --- |
| RCS-2000 | localhost:8182 (rcms REST API) |
| RCA Middleware | 0.0.0.0:8990 (Dashboard / API / PDA) |
| ESP32-S3 | Static IP per station, WiFi 2.4GHz |
| PDA | WiFi 2.4GHz, Chrome browser → http://IP:8990/pda |

## 2.3 Data Flow: Button-Call (MF01)

- Worker presses button (DI1) on ESP32-S3. Event enqueued via FreeRTOS xQueue (≈1µs).

- Core 0 sends POST /api/v1/beeper/press to middleware.

- Middleware validates state (IDLE only), calls RCS genAgvSchedulingTask with 6 positions.

- RCS assigns AMR. Callbacks (start/end) update station state + DB. LED reflects status.

- Station auto-resets to IDLE after completion (10s hold).

## 2.4 Data Flow: PDA Dispatch (MF02)

- Kitting worker selects zone (K1/K2/K3) and destination station on PDA.

- Worker scans position QR (e.g., K13). App validates against active dispatches, auto-advances.

- Worker scans rack QR. App auto-advances to confirmation with 3s countdown.

- Middleware sends genAgvSchedulingTask with MF02 and 2 positions (source → buffer).

- Task status synced via agvCallback + queryTaskStatus polling. DB updated in real-time.

- Worker can cancel active dispatch from PDA history page.

# 3. ESP32-S3 Beeper Thin Client

## 3.1 Hardware

| Spec | Detail |
| --- | --- |
| Board | Waveshare ESP32-S3-POE-ETH-8DI-8DO |
| MCU | ESP32-S3 Xtensa LX7 Dual-Core @ 240MHz |
| Memory | 16MB Flash + 8MB OPI PSRAM |
| DI | 8-ch opto-isolated: DI1=Button, DI8=AP switch |
| DO | 8-ch TCA9554 I2C expander: DO1=Status LED |
| Audio | Buzzer GPIO46 (1-5 beeps for events) |
| Network | WiFi 2.4GHz (STA+AP), Ethernet POE, RS485, CAN |
| Power | 7-36V DC / USB-C / POE |

## 3.2 FreeRTOS Dual-Core Architecture

| Core | Task | Function |
| --- | --- | --- |
| Core 1 | loop() — Main | Button read, LED update, buzzer, web server, DI8 mode check |
| Core 0 | networkTask() | HTTP POST/GET to middleware, state polling, WiFi reconnect watchdog |

Button press → xQueueSend (non-blocking, ~1µs) → Core 0 picks up and sends HTTP. No blocking of main loop.

## 3.3 LED & Buzzer Feedback

| LED Pattern | State | Buzzer | Meaning |
| --- | --- | --- | --- |
| OFF | idle | — | Ready to call |
| SOLID | created | 1 beep | Task created, waiting AMR |
| SLOW (1s) | executing | — | AMR transporting |
| FAST (200ms) | complete | 2 beeps | Delivery done |
| ERROR (100ms) | error | 3 beeps | Failed, auto-reset 15s |

## 3.4 AP Mode Recovery

DI8 switch ON → device reboots as Access Point (SSID: Rozitek, Pass: Rozitek@123). Config at http://192.168.4.1/config. Save → switch DI8 OFF → restarts in STA mode.

# 4. RCA Middleware

## 4.1 Design

- Zero external dependencies — Python 3.6+ stdlib only (no pip install)

- Single-file core: main.py (679 lines) + config.json + pda.html

- Co-located with RCS on HikvisionOS (air-gapped, no internet)

- ThreadedHTTPServer with watchdog, safe_http_post wrapper, full try/except

## 4.2 Technology Stack

| Layer | Technology | Notes |
| --- | --- | --- |
| Language | Python 3.6+ | Pre-installed on HikvisionOS |
| HTTP Server | ThreadedHTTPServer | ThreadingMixIn, 50 queue, daemon threads |
| HTTP Client | urllib.request | safe_http_post() — never raises |
| Database | sqlite3 (stdlib) | dispatch_history + station_events, thread-safe lock |
| Dashboard | Inline HTML/CSS/JS | Server-rendered, 4s auto-refresh, flow layout |
| PDA App | Static HTML served from /pda | Scanner buffer, keypad nav, history+cancel |

## 4.3 State Machine (per station)

| State | Trigger | Next | Auto-Reset |
| --- | --- | --- | --- |
| IDLE | Button / PDA dispatch | CREATED | — |
| CREATED | agvCallback: start | EXECUTING | 600s → ERROR |
| EXECUTING | agvCallback: end | COMPLETE | 600s → ERROR |
| COMPLETE | Timer 10s | IDLE | Auto 10s |
| ERROR | Timer 15s | IDLE | Auto 15s |

## 4.4 SQLite Database

### 4.4.1 dispatch_history

| Column | Type | Description |
| --- | --- | --- |
| id | INTEGER PK | Auto-increment ID |
| time | TEXT | Timestamp YYYY-MM-DD HH:MM:SS |
| kitting | TEXT | KITTING-1/2/3 |
| source | TEXT | Kitting position (K11-K36) |
| rack | TEXT | Rack barcode from QR scan |
| station | TEXT | Destination (T1-T21) |
| buffer | TEXT | Buffer position (T1B-T21B) |
| task_type | TEXT | MF01 or MF02 |
| task_id | TEXT | RCS task UUID |
| state | TEXT | created / executing / complete / cancelled / error |
| message | TEXT | Status message |

State sync: agvCallback updates on event, queryTaskStatus polls every 30s (including orphan tasks from DB), PDA cancel updates directly. State "cancelled" replaces old "error" for cancel actions.

## 4.5 Rack Usage Tracking

Each station tracks how long a trolley is used: deliver_time (task complete) to press_time (button press). Dashboard shows live timer per station with color coding: green <30m, yellow 30-60m, red >60m.

# 5. PDA Kitting Application

## 5.1 Device: Hikvision DS-MDT302

Industrial PDA with touchscreen, physical keypad (0-9 + Enter + Esc), and integrated QR/barcode hardware scanner. Runs Chrome on Android. PDA app served at http://server:8990/pda.

## 5.2 Workflow

| # | Action | Input | Advance |
| --- | --- | --- | --- |
| 1 | Select kitting (K1/K2/K3) | Keypad 1-3 / touch | → Step 2 |
| 2 | Select destination station | Keypad 1-9 / touch (busy blocked) | → Step 3 |
| 3 | Scan position QR at kitting | Hardware scanner (auto-detect) | Auto → Step 4 (800ms) |
| 4 | Scan rack QR code | Hardware scanner (auto-detect) | Auto → Send (3s countdown) |

## 5.3 Scanner Buffer

Hardware scanner sends rapid keystrokes to Chrome. Global keystroke buffer captures characters arriving <80ms apart, distinguishing scanner from manual typing. On scan complete (Enter or 250ms timeout with 2+ chars), value is placed into active input field automatically.

## 5.4 Position & Station Blocking

PDA queries GET /api/v1/kitting/active before rendering position scan and station selection steps. Positions with active dispatches are rejected with alert. Stations with active tasks (button-call or PDA dispatch) are greyed out. Server-side validation also rejects duplicate dispatches.

## 5.5 Cancel from PDA

History page shows all dispatches with state badges. Active dispatches (created/executing) display a Cancel button. Cancel action: (1) POST /api/v1/kitting/cancel with dispatch_id, (2) middleware cancels task on RCS via cancelTask API, (3) DB state updated to "cancelled", (4) station reset to IDLE.

# 6. REST API Reference

| Endpoint | Method | Description |
| --- | --- | --- |
| POST /api/v1/beeper/press | POST | ESP32 button press (MF01) |
| GET /api/v1/beeper/{id}/state | GET | ESP32 poll LED state |
| POST /api/v1/rcs/agvCallback | POST | RCS task status callback |
| POST /api/v1/kitting/dispatch | POST | PDA dispatch (MF02, 2 positions) |
| POST /api/v1/kitting/cancel | POST | Cancel dispatch (RCS + DB + station) |
| GET /api/v1/kitting/active | GET | Active dispatches by position/station |
| GET /api/v1/kitting/history | GET | Dispatch history from SQLite |
| GET /api/v1/pda/config | GET | Kitting layout + stations for PDA |
| GET /api/v1/admin/stations | GET | All stations with state/buffer/usage |
| GET /api/v1/admin/overview | GET | System summary stats |
| POST /api/v1/admin/reset/{id} | POST | Force reset station + cancel DB |
| POST /api/v1/admin/cancel/{id} | POST | Cancel RCS task + reset station |
| GET /pda | GET | Serve PDA web app |
| GET /health | GET | Health check |

# 7. Station & Position Layout

| Zone | Positions | Stations | Buffers |
| --- | --- | --- | --- |
| KITTING-1 (K1) | K11–K16 | T1→T3, T4→T6 | T1B–T6B |
| KITTING-2 (K2) | K21–K26 | T15→T17, T18→T21 | T15B–T21B |
| KITTING-3 (K3) | K31–K36 | T9→T11, T12→T14 | T9B–T14B |

# 8. RCS-2000 V3.3 Integration

## 8.1 CMS Configuration

- Application Registration: No=RCA_BEEPER, IP=127.0.0.1, Port=8990, Path=/api/v1/rcs, Method=REST

- Task Template MF01+MF02: Notify Third-Party → Application=RCA_BEEPER, Method=agvCallback

## 8.2 DB Sync

Background poll queries RCS queryTaskStatus for all active tasks including orphans from DB. RCS status 5=cancelled updates DB, unblocking positions/stations. This ensures DB converges with RCS within 30s even if agvCallback was missed or task was cancelled directly on RCS web.

# 9. Deployment

| File | Size | Function |
| --- | --- | --- |
| main.py | 679 lines | Complete middleware: server, RCS client, state machine, DB, dashboard |
| pda.html | 477 lines | PDA app: 4-step wizard, scanner buffer, history, cancel |
| config.json | 207 lines | 19 stations, 3 kitting zones, task types, RCS connection |
| rca_beeper.db | Auto-created | SQLite: dispatch history + events (persists across restarts) |

nohup python3 /opt/rca_beeper/main.py > /var/log/rca_beeper.log 2>&1 &

# 10. Reliability & Fault Tolerance

| Failure | Detection | Recovery |
| --- | --- | --- |
| ESP32 WiFi loss | Watchdog 10s | Auto-reconnect; DI8 AP fallback |
| RCS unreachable | HTTP timeout 10s | ERROR state; auto-reset 15s |
| Missed agvCallback | Poll every 30s | DB + station synced from RCS |
| Task cancelled on RCS | Poll detects status=5 | DB → cancelled; position unblocked |
| DB orphan records | DB poll finds stale task_ids | Query RCS; update DB state |
| Middleware crash | systemd Restart=always | Restart 5s; SQLite persists |
| Background crash | Internal watchdog | Thread auto-restart 5s |
| Button spam | State ≠ IDLE check | Reject; buzzer feedback |
| PDA position conflict | DB active query + server check | Block + alert + reject dispatch |
| Server power loss | systemd on boot | IDLE reset; DB history preserved |